IMPORTANT INFORMATION - MUST READ

Please note: Before installing this adapter you must determine that this plate will work on your particular engine.

Some engines use an internal gear instead of an external gear and housing, this adapter will not work with those engines unless the gear is removed and modifications are made. Please contact Raven Aircraft for further info. If the gear is left in place you will have zero oil pressure when inverted, secondly oil pressure will not come back when returning to upright flight due to the vacuum applied to the inverted valve by the engine oil pump. If the internal gear is to be removed the lubricating oil passage must be blocked or the oil pressure will be low at all times. However the large opening has to remain open otherwise no oil will reach the plate.

In order for this plate to work oil must be able to freely reach the housing with no gear blocking the passage.

The vast majority of engines have a separate gear housing and gear that are easily removed.

For those engines remove the existing gear and gear housing as the Vac plate adapter must sit right against the accessory case itself or no oil will reach the valve, which will result in zero oil pressure. Again oil pressure will not come back when returning to upright flight due to the vacuum applied to the inverted valve by the engine oil pump.

After removing the gear housing and gear you can take up the resulting 1- ¼” or so space on the studs by fabricating spacers out of tubing or stacking multiple washers.

If you have any questions or concerns please contact us before proceeding with the install.        ravenair@uniserve.com   (250) 495 3863